|  | The Center for Municipal Solutions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Propagation Study Data Sheet |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Applicant: |  |  | Name/title person completing form: |  |  |  |  |  |  |  |  |  |
|  | Proposed Site Name: |  |  | Site Address: |  |  |  |  |  |  |  |  |  |
|  | Instructions: Complete this form, attach radio and antenna spec sheets and include with the RF Propagation Study. All nearby sites (adjacent) providing coverage in and near the proposed site are to be included in the study. Use an additional form if more than 3 sites |  |  |  |  |  |  |  |  |  |  |  |  |
| Line No. | ITEM | UNIT | Proposed site Data | Adjacent site #1 Data | Adjacent site #2 Data | Adjacent site #3 Data |  |  |  |  |  |  |  |
|  | GENERAL INFO: |  |  |  |  |  |  |  |  |  |  |  |  |
| #1 | Site Name |  | Jemez Springs | Pajarito Peak | San Ysidro |  |  |  |  |  |  |  |  |
| #2 | Site Address |  | 17816 Highway 4 Jemez Springs, NM | 659 Pajarito Peak San Ysidro, NM | 24200 Hwy 550 San Ysidro, NM |  |  |  |  |  |  |  |  |
| #3 | Tower or structure height | feet | 62’ Branches 67’ | 150 | 130 |  |  |  |  |  |  |  |  |
| #4 | Antenna mounting height agl | feet | 60’ Mount | 122 | 110 |  |  |  |  |  |  |  |  |
| #5 | Network Technology |  | UMTS | UMTS | UMTS |  |  |  |  |  |  |  |  |
| #6 | Operating Frequency | MgHz | 1956.25 | 1956.25 | 1956.25 |  |  |  |  |  |  |  |  |
| #7 | Base Station Manufacturer |  | ZTE | ZTE | ZTE |  |  |  |  |  |  |  |  |
| #8 | Base Station Model # |  | R8882 | R8882 | R8882 |  |  |  |  |  |  |  |  |
| #9 | Radio Max Power | Watts | 30 (for UMTS) | 30 (for UMTS) | 30 (for UMTS) |  |  |  |  |  |  |  |  |
| #10 | Is pilot channel used for propagation (yes or no) |  |  |  |  |  |  |  |  |  |  |  |  |
| #11 | If yes, pilot channel power (min. 20% of power) | Watts | 6 | 6 | 6 |  |  |  |  |  |  |  |  |
| #12 | Convert Power to dBm | dBm | 37.8 | 37.8 | 37.8 |  |  |  |  |  |  |  |  |
| #13 | Losses: |  |  |  |  |  |  |  |  |  |  |  |  |
| #14 | Is there a combiner |  | No | No | No |  |  |  |  |  |  |  |  |
| #15 | If yes make and model |  |  |  |  |  |  |  |  |  |  |  |  |
| #16 | If yes, combiner loss | dB |  |  |  |  |  |  |  |  |  |  |  |
| #17 | Cable losses: |  |  |  |  |  |  |  |  |  |  |  |  |
| #18 | Cable diameter | In. | N/A (fiber) | 7/8” | 7/8” |  |  |  |  |  |  |  |  |
| #19 | Cable lgth. From antenna to equipment | Ft | N/A | 142 | 130 |  |  |  |  |  |  |  |  |
| #20 | Cable loss per 100 ft. | dB | N/A | 1.863 | 1.836 |  |  |  |  |  |  |  |  |
| #21 | Calculated loss | dB | 0 | 2.65 | 2.42 |  |  |  |  |  |  |  |  |
| #22 | Connectors |  | 2 | 4 | 4 |  | #23 | Number of connectors |  |  |  |  |  |
| #24 | Loss per connector | dB | .2 | 0.2 | 0.2 |  |  |  |  |  |  |  |  |
| #25 | Calculated Loss | dB | .4 | 0.8 | 0.8 |  |  |  |  |  |  |  |  |
| #26 | Jumpers: |  |  |  |  |  |  |  |  |  |  |  |  |
| #27 | Number of jumpers |  | 1 | 2 | 2 |  |  |  |  |  |  |  |  |
| #28 | Loss per jumper | dB | .3 | 0.3 | 0.3 |  |  |  |  |  |  |  |  |
| #29 | Calculated loss | dB | .3 | 0.6 | 0.6 |  |  |  |  |  |  |  |  |
| #30 | Total calculated loss: | dB | 0.9 | 4.05 | 3.82 |  |  |  |  |  |  |  |  |
| #31 | Gains: |  |  |  |  |  |  |  |  |  |  |  |  |
| #32 | Is there an in line amplifier |  | No | No | No |  |  |  |  |  |  |  |  |
| #33 | If yes, gain | dB |  |  |  |  |  |  |  |  |  |  |  |
| #34 | Antenna Mftg. |  | Amphenol | Amphenol | Amphenol |  |  |  |  |  |  |  |  |
| #35 | Antenna Model # |  | BXQ185063/12CF | BXA185090/8CF | BXA185090/8CF |  |  |  |  |  |  |  |  |
| #36 | Antenna gain | dB | 20 | 16.5 | 16.5 |  |  |  |  |  |  |  |  |
| #37 | Total gain: | dB | 20 | 16.5 | 16.5 |  |  |  |  |  |  |  |  |
| #38 | Result: |  |  |  |  |  |  |  |  |  |  |  |  |
| #39 | Transmission Power | dBm | 64.1 | 57.3 | 57.1 |  |  |  |  |  |  |  |  |
| #40 | ERP | Watts | 2570 | 537 | 513 |  |  |  |  |  |  |  |  |
|  | To Calculate ERP (Line #40) of an Antenna: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Step #1: Insert the radio power in Watts in line 9 and pilot power, if applicable in Line 11. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Step #2: Convert the radio power or pilot power, as applicable, into dBm units. Insert the result into Line #12 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Step #3: Add all dB losses: Line #16 + Line #21+ Line #25 + Line #29; Insert the result into Line #30 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Step #4: Add all dB gains: Line #33 + Line #36; Insert the result into Line #37 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Step $5: Add Line #37 to Line #12. Subtract from the result, Line #30. Insert the result into Line #39. |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Step #6: Convert the number in Line #39 into Watts. Record the result in Line #40. |  |  |  |  |  |  |  |  |  |  |  |  |